L-804(L) LED RUNWAY GUARD LIGHT - SAMPLE SPECIFICATION

Note: Modify the items in italics according to your specific job requirements.

ITEM L-125 INSTALLATION OF L-804(L) LED LIGHTS

DESCRIPTION

125-1.1 This item shall consist of furnishing and installing the L-804(L) LED fixtures in accordance with these specifications.

This item shall also include all wire and cable connections, the furnishing and installing of all necessary conduits and fittings and all necessary mounting structures. It shall also include the testing of the installation and all incidentals necessary to place the lights in operation as completed units to the satisfaction of the Engineer.

EQUIPMENT AND MATERIALS

125-2.1 L-804(L).  The L-804(L) fixtures shall conform to the requirements of FAA Advisory Circular 150/5345-46 (current edition) “Specification for Runway and Taxiway Light Fixtures” and FAA LED “Engineering Brief No. 67” (current edition).  The L-804(L) LED shall be ETL certified. The L-804(L) fixtures shall be as manufactured by AIRPORT LIGHTING COMPANY (www.airportlightingcompany.com) or approved equal.

125-2.2 EQUIPMENT SUPPLIED.  FAA L-804(L) fixtures shall be supplied for the total quantity as shown on the plans.  Each lot shipment of fixtures shall include one Instruction Manual.  The manufacturer shall also have a downloadable electronic version of the manual available on their web site.

125-2.3 L-804(L) LED FIXTURE.  To ensure reduced maintenance requirements and increased pilot conspicuity, the L-804(L) fixture light source shall be a Light Emitting Diode (LED) assembly.  The fixture shall have an alternating flash rate of 45-50 flashes per minute.  The required aviation yellow beam color shall be obtained without the use of a color filter.

The L-804(L) optics shall be held by a single frangible column that allows the fixture to be easily aimed from 0° to 20 vertically and ±20 horizontally.  The two RGL light sources shall be surrounded by a black face plate and independent visors to maximize the contrast during the ON/OFF cycle.  Access to the electronic control device shall be achievable through a waterproof front panel on the flash head to permit easy replacement of the controller.

The fixture shall be fabricated from corrosion-resistant material and all exterior surfaces shall be powder coated aviation yellow for additional protection and visibility.  The elevated RGL fixture shall be mounted using a high-strength ALC RGL-6 base plate.

(ALC is instant on, instant off only.)

125-2.5 L-804 ELECTRICAL.  {The LED L-804(L) fixture shall be designed to operate in FAA Mode 1, series circuit operation. The LED L-804(L) shall operate on a 6.6A secondary, 50/60Hz isolation transformer. The fixture load shall be 59VA maximum and shall use a 6.6A/6.6A, 65W isolation transformer. Lamp intensity shall vary with current supplied to the fixture by the series circuit.  The fixture light output shall match quartz-incandescent fixtures at all brightness levels on either a 3-step and 5-step regulator.  It shall be possible to install the lights on existing circuits in addition to, or in replacement of, conventional incandescent or halogen lights without having to change any other element (CCR, primary and secondary cabling, series transformer, etc.). }

or

{The LED L-804(L) fixture shall be designed to operate in FAA Mode 2, voltage driven operation.  The light fixture shall operate from either a 120Vac ±10%, 50/60Hz or a 240Vac ±10%, 50/60Hz parallel lighting circuit and shall be equipped with a photocell to control lamp intensity.  The photocell shall energize the fixture at full intensity during high light levels and then reduce intensity to 30% during low ambient light conditions.  The fixture light output shall match quartz-incandescent fixtures at both intensity steps.   It shall be possible to install the lights on existing circuits in addition to, or in replacement of, conventional incandescent or halogen lights without having to change existing power wiring.  The power consumption of a Mode 2 L-804 is 34.7 Watts at full intensity.

{The LED L-804(L) shall be unmonitored.}

or

{The LED L-804(L) shall be provided with dedicated fault monitoring contacts (one for each LED window) via a multi-pin plug.  This shall provide an external dry contact closure to indicate alarm status.}

CONSTRUCTION METHODS

125-3.1 PLACING THE LED L-804(L) FIXTURES.  The contractor shall furnish and install each LED L-804(L) fixture as specified in the proposal and shown in the plans.  The L-804(L) fixtures shall be mounted on a high-strength ALC RGL-6 base plate.  The base plate shall then be mated to a standard FAA L-867 base can at the locations shown on the plans.

125-3.2 TESTS.  All L-804(L) LED fixtures shall be fully tested by continuous operation for not less than 24 hours as a completed system prior to acceptance.  {For Mode 1 powered L-804(L)s, the test shall include operating the constant current regulator in each step not less than 10 times at the beginning and end of the 24-hour test.  The L-804(L) shall illuminate properly and have the proper flash timing during each portion of the test.}  {For Mode 2 powered L-804(L)s, the test shall include operating the power source on and off not less than 10 times at the beginning and end of the 24-hour test.  Proper operation of the photocell shall also be verified.  The L-804(L) shall illuminate properly and have the proper flash timing during each portion of the test.}

METHOD OF MEASUREMENT

125-4.1 MEASUREMENT.  The quantity of lights to be paid for under this item shall be for the total quantity of L-804(L) LED fixtures installed and one Instruction Manual (per lot) and accepted as completed units, in place, ready for operation.

BASIS FOR PAYMENT

125-5.1 PAYMENT.  Payment will be made at the contract unit price for the completed total quantity of L-804(L) LED fixtures installed, in place by the Contractor, and accepted by the Engineer. This price shall be full compensation for furnishing all materials and for all preparation, assembly, and installation of these materials, and for all labor, equipment, tools, and incidentals necessary to complete this item.

Payment will be made under:

Item L-125-5.1 	L-804(L) LED fixtures, in Place—per each

END OF ITEM L-125